| Permit Ref. No.: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ## HOT WORK PERMIT |  |  |  |  |
| THIS HOT WORK PERMIT IS REQUIRED FOR ANY OPERATION INVOLVING OPEN FLAMES OR PRODUCING HEAT AND/OR SPARKS. THIS INCLUDES: WELDING, CUTTING, BRAZING, GRINDING, AND SOLDERING AND TORCH APPLIED ROOFING. |  |  |  |  |  |  |
| CONTRACTOR: |  |  | LOCATION: |  |  |  |
| TYPE OF HOT WORK & JOB DESCRIPTION: |  |  | PERMIT PERIOD: to |  |  |  |
| EQUIPMENT USED: |  |  | EMERGENCY PHONE NO: |  |  |  |
| PPE REQUIREMENTS: Face shield, Air plug or muffs, safety boots and goggles, welding helmets, welding gloves and apron. |  |  | PERMIT ISSUED BY (TECH Manager): ………………………………. (Name) (signed) |  |  |  |
| No. | PRECAUTION/ HAZARD CONTROL REQUIREMENT |  |  |  | YES | N/A |
| 1 | Smoke and heat detectors and fire alarm systems to be isolated for duration of the work. |  |  |  |  |  |
| 2 | Affected services to be isolated; equipment to be locked-out or tagged-out (SHE/P1/26). |  |  |  |  |  |
| 3 | Hot work equipment to be in good order and inspected prior to use (SHE/P1/39). |  |  |  |  |  |
| 4 | Gas cylinders to be >5m from heat source; have flash back arrestors and non-return valves fitted (SHE/P1/03). |  |  |  |  |  |
| 5 | Flammable materials/ liquids to be removed from the area (11m clear – GS220). Area to be clean. |  |  |  |  |  |
| 6 | Fire resistant curtains/ shields to be used to protect the public/ site staff (work min. 1.5m from other persons). |  |  |  |  |  |
| 7 | Fire resistant curtains/ shields to be used where required i.e. erected at sides and below in order to contain sparks. |  |  |  |  |  |
| 8 | Fire resistant blankets to be used where required e.g. openings to other levels. |  |  |  |  |  |
| 9 | Adequate ventilation to be provided and satisfactory access/ egress from the area to be ensured. |  |  |  |  |  |
| 10 | Access to the work area to be controlled i.e. barriers and signage erected if required. |  |  |  |  |  |
| 11 | Fire extinguishing equipment to be maintained in a state of readiness. |  |  |  |  |  |
| 12 | Fire watch trained and stationed nearby. Fire watch to be maintained for at least a half hour after hot work. |  |  |  |  |  |
| Prior to hot work | CONFIRMATION BY SUPERVISOR: I confirm that the persons carrying out this work are competent to perform this work and are fully briefed on the safe method of work, and that I will ensure that the precautions noted above will be complied with. …………………………………………… ………………………………………………….. …………………………………... (name) (signed) (date) |  |  |  |  |  |
|  | CONFIRMATION BY OPERATOR: I will comply with the PPE requirements and precautions while carrying out this hot work. …………………………………………… ………………………………………………….. …………………………………... (name) (signed) (date) |  |  |  |  |  |
| After completion of hot work | CANCELLATION BY OPERATOR: I confirm that the work has been completed and that the area has been checked and is safe. …………………………………………… ………………………………………………….. …………………………………... (name) (signed) (date) |  |  |  |  |  |
|  | INSPECTION OF AREA BY SUPERVISOR: The area has been inspected after a half hour of completing the work and is safe. …………………………………………… ………………………………………………….. …………………………………... (name) (signed) (date) |  |  |  |  |  |
|  | CANCELLATION BY SAFETY OFFICER: …………………………………………… ………………………………………………….. …………………………………... (name) (signed) (date) |  |  |  |  |  |